# Gazebo and Ardupilot SITL Setup Guide

For using first time  Gazebo and SITL ardupilot we should follow these steps.

1.first we should install the hormonic Gazebo with its own plugins .

sudo apt update

sudo apt install libgz-sim8-dev rapidjson-dev

sudo apt install libopencv-dev libgstreamer1.0-dev libgstreamer-plugins-base1.0-dev gstreamer1.0-plugins-bad gstreamer1.0-libav gstreamer1.0-gl

2. clone the Ardupilot _gazeo  and build it .

git clone https://github.com/ArduPilot/ardupilot_gazebo

cd ardupilot_gazebo

mkdir build && cd build

cmake .. -DCMAKE_BUILD_TYPE=RelWithDebInfo

make -j4

3. Set the Gazebo path (the simulation pluging,the models and the worlds) environment variables in your .bashrc .

3.1 this is for the simulation path it should be correct based on the arduiplilot gazebo build   path .

echo 'export GZ_SIM_SYSTEM_PLUGIN_PATH=$HOME/Ard_Gaze/ardupilot_gazebo/build  $GZ_SIM_SYSTEM_PLUGIN_PATH'

3.2 this is for the ardupilot models and worlds it should be also correct path or the worlds and models like the bellow .

echo 'export GZ_SIM_RESOURCE_PATH=$HOME/Ard_Gaze/ardupilot_gazebo/models:$HOME/Ard_Gaze/ardupilot_gazebo/worlds'

source ~/.bashrc

3.4 for the checing of the correct path or not use this .

nano ~/.bashrc

4. after that we can run the Gazebo and Ardupilot SITL .

4.1  for the gazeo strat

gz sim -v4 -r iris_runway.sdf

4.2 for the Ardupliot SITL .

Tools/autotest/sim_vehicle.py -v ArduCopter -f gazebo-iris   --model JSON   --out=udp:127.0.0.1:14550   --out=udp:127.0.0.1:14551   --map --console

5.for the Removing and checking the gazebo cache  we use this .

ps aux | grep gz

6. we can use this website for more reference and more features .

https://github.com/ArduPilot/ardupilot_gazebo

7. for the enabling the gazeo Camera .

7.1 we can enable that with the help of the Gestremer  and we can configure the camera setting in the gazebo models gimble 3D modle.sdf file  .

<plugin name="GstCameraPlugin"

filename="GstCameraPlugin">

<udp_host>127.0.0.1</udp_host>

<udp_port>5602</udp_port>  ----  the port we forward the display

<use_basic_pipeline>true</use_basic_pipeline>

<use_cuda>false</use_cuda>

</plugin>

7.2 for enabling or on  the camera display before the camera.

gz topic -t /world/gimbal/model/mount/model/gimbal/link/pitch_link/sensor/camera/image/enable_streaming -m gz.msgs.Boolean -p "data: 1"     ... or run the file of the gazebo_camera_on.py in the  Frame-base-Tracking project .

7.3  for the checking the stream is working or not  after enabling the the camera .

~$ gst-launch-1.0 -v udpsrc port=5602  caps='application/x-rtp,media=video,encoding-name=H264,payload=96,clock-rate=90000' ! rtpjitterbuffer latency=100 ! tee name=t ! queue ! udpsink host=127.0.0.1 port=5604 t. ! queue ! udpsink host=127.0.0.1 port=5605 .

7.4    for camera diaplay use file of the test_camera_stream.py in the Frame-base-Tracking project .

7.5 for the yaw and pitch of the Gazebo camera use this .

cd ardupilot sim_vehicle.py -D -v ArduCopter -f JSON --add-param-file=$HOME/ardupilot_gazebo/config/gazebo-iris-gimbal.parm --console --map